Assignment – 4

Q1. An assembly program to add two numbers and store the 8 bit sum in any general purpose register.

MVI A, 7
MVI B, 5
ADD B
MOV C, A
HLT

+-------+---------+------------+---------------+----------------+----------+

|Address|Mnemonics|Machine Code|Addressing Mode|Instruction Type|Operation |

+-------+---------+------------+---------------+----------------+----------+

|4200   |MVI A, 7 |3E 07       |Immediate      |2 bytes         |A <- 7    |

|4202   |MVI B, 5 |06 05       |Immediate      |2 bytes         |B <- 5    |

|4204   |ADD B    |80          |Register       |1 bytes         |A <- A + B|

|4205   |MOV C, A |4F          |Register       |1 bytes         |C <- A    |

|4206   |HLT      |76          |null           |1 bytes         |null      |

+-------+---------+------------+---------------+----------------+----------+

Q2. An assembly program to add two numbers and store the 8 bit sum in any Memory location.

MVI A, 7
MVI B, 5
ADD B
STA 0
HLT

+-------+---------+------------+---------------+----------------+----------+

|Address|Mnemonics|Machine Code|Addressing Mode|Instruction Type|Operation |

+-------+---------+------------+---------------+----------------+----------+

|4200   |MVI A, 7 |3E 07       |Immediate      |2 bytes         |A <- 7    |

|4202   |MVI B, 5 |06 05       |Immediate      |2 bytes         |B <- 5    |

|4204   |ADD B    |80          |Register       |1 bytes         |A <- A + B|

|4205   |STA 0    |32 00 00    |Indirect       |3 bytes         |C <- A    |

|4206   |HLT      |76          |null           |1 bytes         |null      |

+-------+---------+------------+---------------+----------------+----------+

Q3. An assembly program to subtract two numbers and store the result in any Memory location.

MVI A, 7
MVI B, 5
SUB B
STA 0
HLT

+-------+---------+------------+---------------+----------------+----------+

|Address|Mnemonics|Machine Code|Addressing Mode|Instruction Type|Operation |

+-------+---------+------------+---------------+----------------+----------+

|4200   |MVI A, 7 |3E 07       |Immediate      |2 bytes         |A <- 7    |

|4202   |MVI B, 5 |06 05       |Immediate      |2 bytes         |B <- 5    |

|4204   |SUB B    |90          |Register       |1 bytes         |A <- A - B|

|4205   |STA 0    |32 00 00    |Indirect       |3 bytes         |C <- A    |

|4206   |HLT      |76          |null           |1 bytes         |null      |

+-------+---------+------------+---------------+----------------+----------+