Assignment 3

Q1. Set up a network with router in GUI and CLI.

1. Set up a network with router in GUI

- Create one router, two switches, and ten PCs in Cisco Packet Tracer.

- Connect the PCs to the switches and connect both switches to the router using Copper Straight-Through cables.

- Configure the router interfaces and assign IP addresses through the GUI.

- Configure the IP addresses, subnet masks, and default gateways of all PCs.

- Test communication between PCs using Simple PDU in Simulation Mode.

2. Set up a network with router in CLI

- Create one router, two switches, and ten PCs in Cisco Packet Tracer.

- Connect the PCs to the switches and connect both switches to the router using Copper Straight-Through cables.

- Configure the router interfaces using CLI commands and assign IP addresses.

- Configure the IP addresses, subnet masks, and default gateways of all PCs.

- Test communication between PCs using Simple PDU in Simulation Mode.

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface GigabitEthernet0/0/0

Router(config-if)#ip address 192.168.10.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up

exit

Router(config)#interface GigabitEthernet0/0/1

Router(config-if)#ip address 192.168.11.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/1, changed state to up

exit

Router(config)#exit

Router#

%SYS-5-CONFIG_I: Configured from console by console

Exit