Assignment 4

Q1. Configure DHCP Services with a Server.

- Create one Server-PT, one Cisco 2960 switch, and five PCs in Cisco Packet Tracer.

- Connect the server and all PCs to the switch using Copper Straight-Through cables.

- Click Server0 → Desktop → IP Configuration and assign the static IP address 192.168.1.2, subnet mask 255.255.255.0, and default gateway 192.168.1.1.

- Click Server0 → Services → DHCP, turn the DHCP service On, and configure the following settings:

- Pool Name: LAN

- Default Gateway: 192.168.1.1

- DNS Server: 8.8.8.8

- Start IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Maximum Number of Users: 50

- Click Add or Save to create the DHCP pool.

- For each PC, open Desktop → IP Configuration and select DHCP to obtain an IP address automatically.

- Switch to Simulation Mode and filter for DHCP packets to observe the four stages of the DORA process.

DHCP DORA Process

- Discover: The PC broadcasts a DHCP Discover packet to find a DHCP server.

- Offer: The server offers an available IP address to the PC.

- Request: The PC requests the offered IP address.

- Acknowledge: The server confirms the assignment, and the PC receives its network configuration.

Discover Packet

Offer Packet:

Request Packet:

Acknowledge Packet: